VIVA VOCE COMPREHENSIVE TECHNICAL DEFENSE & MASTERY REPORT

Safety-Constrained Adaptive SDN Mitigation and Monitoring for IoMT Networks

| Student Name: | H.M.S.H. Wijerathna |
| --- | --- |
| Student ID: | IT23306936 |
| Specialization: | BSc (Hons) in Information Technology - Computer Systems & Network Engineering (CSNE) |
| Project Code & Title: | J26-CS-330 — Lightweight SDN-Based Intrusion Detection and Mitigation System for IoMT Networks |
| Individual Component: | Component 2: Safety Constrained Adaptive SDN Mitigation and Real-Time QoS Telemetry Monitoring |

EXECUTIVE VIVA GUIDE SUMMARY This master document serves as your exhaustive, publication-grade defense manual for your undergraduate viva voce examination. It provides deep architectural analysis, mathematical formulations, concrete packet accounting proofs, code trace references, reproducible benchmark evidence, and 25 rigorously formulated examiner Q&As covering foundational, edge-case, and architectural scrutiny.

1. Research Context, Problem Formulation & Engineering Motivation

1.1 The IoMT Vulnerability Paradox

Modern healthcare infrastructure relies heavily on the Medical Internet of Things (IoMT)—wearable biosensors, continuous electrocardiogram (ECG) telemetry, smart infusion pumps, and patient monitoring stations. These devices operate under extreme asymmetry: they possess severely constrained computing, memory, and energy budgets, rendering onboard cryptographic verification, deep packet inspection, and client-side endpoint detection agents computationally prohibitive. Consequently, clinical sensors transmit plaintext or minimally protected UDP/TCP streams across shared hospital Ethernet and wireless subnets.

Under volumetric Distributed Denial-of-Service (DDoS) flooding, SYN floods, or UDP amplification attacks, shared bottleneck switches and access points suffer severe buffer exhaustion. Clinical telemetry experiences immediate queue starvation, resulting in intolerable packet loss and transmission latency jitter that destabilizes life-critical patient monitoring.

1.2 The Fatal Flaw of Traditional IT Defenses: Collateral Clinical Denial of Service

Traditional enterprise network security relies on blunt mitigation instruments: static Access Control Lists (ACLs), ingress port blocking, and blanket IP blackholing. In a clinical IoMT deployment, such mechanisms introduce catastrophic collateral damage:

Blanket Port Dropping: Attackers frequently spoof destination ports or target open receiver ports (e.g., edge ingestion servers on UDP 5201/5202). Applying an OpenFlow DROP rule on the destination port terminates malicious traffic but simultaneously destroys legitimate clinical telemetry arriving at that port, achieving the attacker's objective.

Host Blacklisting Under IP Spoofing: When adversarial traffic spoofs internal hospital IPs (or when an attacker launches a Man-in-the-Middle ARP poisoning attack), dropping traffic based on Source IP inadvertently disconnects genuine sensors or medical edge gateways.

Static Rate Limiting Rigidity: Static token buckets configured without continuous telemetry feedback cannot adapt to dynamic clinical bursts or secondary network congestion. If set too loose, medical telemetry Starves; if set too tight, valid administrative or diagnostic flows are unnecessarily throttled.

1.3 The Individual Component Research Objective

To resolve this fundamental paradox, this individual component (IT23306936) formulates, implements, and experimentally validates a Safety-Constrained, Adaptive Software-Defined Networking (SDN) Mitigation and Real-Time QoS Telemetry Monitoring System. The system decouples threat detection from policy enforcement, strictly prioritizes clinical safety constraints over security enforcement, employs OpenFlow 1.3 meters to throttle offending traffic rather than dropping critical channels, continuously evaluates empirical Quality of Service (QoS) through rolling windows, dynamically reassesses mitigation efficacy, and safely cleans up temporary rules through an automated two-window post-threat stabilization gate.

2. End-to-End System Architecture & Component Decomposition

The architecture is partitioned into four decoupled, fail-closed tiers designed to maintain strict modularity, high availability, and deterministic latency overhead:

| Functional Tier | Key Modules / Files | Primary Engineering Responsibilities |
| --- | --- | --- |
| Tier 1: Ingestion & Validation | alert_api_v2.py alert_validation_v2.py controller_detector_interfaces.py | Ingests asynchronous JSON alerts from upstream ML detectors (DoS, Recon, MITM); enforces strict schema validation (HTTP 400, 413, 415); prevents duplicate and in-flight alert collisions via thread-safe FIFO deques (HTTP 409). |
| Tier 2: Service Context & Policy | configs/services_v2.json context_store_v2.py mitigation_policy_v2.py | Maintains trusted service criticality registry (ECG = critical, background = non-critical); validates flow context; computes safety-constrained mitigation plans (METER_BAND vs DROP); evaluates dynamic reassessment rules. |
| Tier 3: Data Plane Enforcement & Telemetry | ovs_enforcement_v2.py qos_window_v2.py delay_reader_v2.py telemetry_contract.py | Executes OpenFlow 1.3 meter rate-limiting (1000 Kbit/s default, 500 Kbit/s adjusted) and flow rules via Open vSwitch; aggregates receiver intervals into rolling 10s QoS windows; samples ICMP RTT latency with half-open windowing. |
| Tier 4: Lifecycle & UI Monitoring | rule_lifecycle_v2.py controlled_lifecycle_test.py dashboard_server.py static/ & templates/ | Governs temporary rule eviction via two-window post-threat stabilization gate; validates canary cookie isolation; streams real-time telemetry over SSE to dark-mode web UI; renders 3D Three.js hospital topology. |

2.1 Detailed Module Breakdown

A. REST Alert Ingestion Engine (alert_api_v2.py & alert_validation_v2.py)

The alert receiver acts as the formal boundary between untrusted detection agents and the SDN data plane. It exposes a REST endpoint (`POST /alert` and `/api/v2/alerts`) running on Flask. To prevent injection attacks and memory exhaustion, the payload is evaluated against four strict gatekeeper criteria:

Content-Type Enforcement: Requires `application/json`; any other MIME type is rejected immediately with HTTP 415 Unsupported Media Type.

Payload Size Boundary: Enforces `MAX_CONTENT_LENGTH = 65,536` bytes (64 KB). Oversized payloads return HTTP 413 Payload Too Large, preventing memory buffer overflow.

Schema Validation: Checks for mandatory fields: `alert_id` (string), `attack_type` (DoS_Flooding, Reconnaissance, MITM, Unknown), `source_ip` (valid IPv4), `target_ip` (valid IPv4), `confidence` (float 0.0 - 1.0), and `timestamp` (ISO 8601 UTC). Malformed types return HTTP 400 Bad Request with explicit error detail.

Duplicate & Concurrency Guard: Maintains thread-locked `in_flight_ids` and bounded FIFO `seen_ids` (capacity: 10,000 entries). Concurrent requests with the same `alert_id` return HTTP 409 Conflict, ensuring zero race conditions during rule derivation.

B. Service Criticality Registry (configs/services_v2.json)

Clinical safety requires that policy engines never guess traffic priority. The registry defines authoritative network bindings:

JSON REGISTRY SCHEMA (configs/services_v2.json) { "ECG_Monitoring": { "criticality": "critical", "target_ip": "10.0.0.2", "protocol": "udp", "port": 5202, "qos": { "min_throughput_kbps": 90.0, "max_loss_percent": 1.0, "max_delay_ms": null } }, "Background_Traffic": { "criticality": "noncritical", "target_ip": "10.0.0.2", "protocol": "udp", "port": 5201, "qos": null } }

Rule Evaluation Invariant: A mitigation action targeting `target_port: 5202` or `ECG_Monitoring` is NEVER permitted to install a DROP rule. Any mitigation applied to protected medical flows must be safety-constrained to preserve clinical continuity.

C. OpenFlow 1.3 Meter Enforcement (ovs_enforcement_v2.py)

OpenFlow 1.3 meters provide hardware- and kernel-level rate-limiting through per-flow meters. Unlike OpenFlow 1.0 (which only supported port drops or queue enqueuing), OpenFlow 1.3 introduces meter tables with configurable bands. Our system constructs an OVS meter with a `drop` band:

1. Meter Creation: `ovs-ofctl -O OpenFlow13 add-meter s1 meter=1,kbps,band=type=drop,rate=1000`
2. Flow Attachment: `ovs-ofctl -O OpenFlow13 add-flow s1 priority=100,cookie=0x1a2b3c...,table=0,udp,nw_src=10.0.0.3,nw_dst=10.0.0.2,tp_dst=5201,hard_timeout=300,actions=meter:1,output:2`
This configuration guarantees that background or attacking traffic (10.0.0.3 -> 10.0.0.2:5201) is metered at exactly 1000 Kbit/s (or adjusted to 500 Kbit/s) inside the switch datapath. Packets exceeding the meter rate are dropped immediately at ingress, completely relieving the shared bottleneck link queue and allowing medical packets (port 5202) to transit without queue delay or buffer overflow.

D. Rolling QoS Windowing & Delay Tracking (qos_window_v2.py & delay_reader_v2.py)

Network telemetry in live environments is highly volatile. Making policy decisions based on instantaneous 1-second samples causes control oscillations (flapping). To provide statistical stability, the system implements a rolling window architecture:

Rolling Window Formulation: Aggregates five contiguous 2-second receiver intervals into a 10-second rolling window (total duration >= 9.9s).

Packet-Count Loss Aggregation: Instead of computing the arithmetic mean of loss percentages (which introduces severe mathematical skew if interval durations differ), loss is calculated from raw packet counters: Loss% = (Total Lost Packets / Total Sent Packets) * 100.

Half-Open ICMP RTT Windowing: The `PeriodicDelayMonitor` transmits continuous ICMP probes, partitioning replies into half-open time intervals [window_start, window_end). Without hardware timestamping on probe dispatches, the monitor honestly reports reply-arrival RTT statistics (min, avg, max) and keeps sent counts and loss strictly as `null` to avoid fraudulent loss claims.

Transition Window Purging: Whenever a rate-limiting rule is installed or adjusted, the immediately following observation window contains a blend of unmitigated and mitigated packets. The system unconditionally purges this 'transition window' from policy decisions, preventing false reassessments.

E. Post-Threat Rule Lifecycle Cleanup Gate (rule_lifecycle_v2.py)

Leaving security rules permanently active in switch flow tables causes flow table bloat, TCAM exhaustion, and permanent throttling of legitimate non-critical services. However, removing rules prematurely upon the first un-congested sample allows attackers to pulse traffic (pulsing DoS). The `CleanupGate` enforces a rigorous state machine:

Threat Cessation Notification: Triggered when upstream detector signals `threat_status: ended` or external API confirms attack subsidence.

Transition Invalidation: Any observation window that overlaps the threat cessation timestamp is invalidated.

Two-Window Stabilization Criterion: Requires exactly two consecutive, fully completed 10-second post-threat observation windows where measured QoS passes all thresholds (Throughput >= 90 Kbit/s, Loss <= 1%).

Targeted Eviction: Upon gate fulfillment, only the specific flow and meter matching the incident's 64-bit random cookie are deleted. Default switch forwarding and canary security rules remain completely untouched.

3. Technologies Used & Architectural Justifications

| Technology / Tool | Role in Prototype | Deep Technical Justification & Why Chosen |
| --- | --- | --- |
| Python 3.9+ / Linux standard libraries | Core Controller & Evaluator | Provides high-performance asynchronous subprocess control, POSIX IPC, thread locking, and robust socket handling without heavyweight runtime dependencies. |
| Open vSwitch (OVS 2.17+ / 3.x) | Software SDN Switch (s1) | Industry-standard multilayer open virtual switch. Provides full support for OpenFlow 1.3 meters, flow table cookies, kernel datapath acceleration, and microsecond-level rule installation. |
| OpenFlow 1.3 Protocol | Southbound Control Protocol | Chosen over OpenFlow 1.0 because 1.0 lacks meter tables and per-flow metering. OF 1.3 allows dynamic meter bands (drop/DSCP remark) that enable rate limiting without packet dropping. |
| Mininet Network Emulator | Virtual Emulation Environment | Provides kernel-isolated network namespaces for realistic multi-host topologies (h1 sensor, h2 edge, h3 attacker, s1 switch). Allows precise Linux traffic control (tc) link shaping. |
| iperf3 3.18 & Linux TC (HTB/netem) | Traffic Generation & Shaping | Generates high-precision, paced UDP offered load (100 Kbit/s sensor, 5000 Kbit/s attacker). HTB creates an authentic 2 Mbit/s bottleneck queue; netem models link propagation latency. |
| Flask & Flask-CORS | REST API & SSE Telemetry Engine | Lightweight, deterministic microframework. Supports Server-Sent Events (SSE) for low-overhead, real-time telemetry streaming to browsers without WebSocket connection complexity. |
| Three.js & Vanilla WebGL / CSS | Clinical 3D Topology Dashboard | Zero-dependency, high-performance 3D visualization. Renders hospital isometric layout, animated SDN packet flows, meter drops, and live telemetry dials without frontend framework bloat. |
| Pytest Test Framework | Automated Unit & Acceptance Suites | Enables offline verification across 131 unit tests and 10 acceptance regression tests with fake monotonic clocks, validating logic independently of physical Mininet hardware. |

4. Mathematical Formulations & Algorithmic Mechanics

4.1 Mathematical Rigor in QoS Calculations

Examiners frequently scrutinize network metrics calculation. In this prototype, metrics are derived with strict mathematical fidelity:

EQUATION 1: TRUE ROLLING WINDOW THROUGHPUT Throughput (R) in Kbit/s over an aggregated window of N intervals: 8 * ∑_{i=1}^{N} Bytes_Received_i R = ----------------------------------- 1000 * Δt_measured Where Δt_measured is the actual elapsed duration (t_end - t_start) in seconds (asserted >= 9.9s), ensuring exact throughput accounting even under clock drift or process scheduling jitter.

EQUATION 2: TRUE PACKET-COUNT LOSS (REJECTING PERCENTAGE AVERAGING) Packet Loss (L) in percent over an aggregated window of N intervals: ∑_{i=1}^{N} Lost_Packets_i L = ----------------------------------- * 100 % ∑_{i=1}^{N} (Received_i + Lost_i) CRITICAL DEFENSE POINT: Many amateur implementations calculate Loss as: (Loss_1 + Loss_2 + ... + Loss_N) / N. That is mathematically invalid if interval durations or packet arrivals vary. Our implementation aggregates raw packet counters across all intervals, guaranteeing 100% mathematical accuracy.

4.2 Dynamic Reassessment & Step-Down Algorithm

The adaptive mitigation policy follows a deterministic bounded state-machine:

Step 1 (Pre-Mitigation Observation): Window 1 (0-10s) captures baseline unmitigated traffic under attack. Sensor throughput degrades to ~23 Kbit/s, loss spikes to ~77%.

Step 2 (Initial Enforcement): At t=10s, API applies default rate-limit meter of 1000 Kbit/s on the offending flow (10.0.0.3 -> 10.0.0.2:5201).

Step 3 (Transition Window Invalidation): Window 2 (10-20s) is purged from policy assessment to allow switch queues to drain.

Step 4 (Efficacy Reassessment at Window 3): At t=30s, the policy inspects Window 3 QoS:
  • If Sensor Throughput >= 90 Kbit/s AND Packet Loss <= 1.0%: Mitigation succeeded. Status = 'no_change_recommended'.
  • If Sensor Loss > 1.0% (degraded QoS): Trigger Secondary Step-Down -> Rate adjusted to 500 Kbit/s.
  • Bounded Invariant: Exactly ONE adjustment is authorized. Unbounded step-downs are forbidden to prevent total throttling.

Step 5 (Post-Adjustment Evaluation): Window 4 (30-40s) is purged as a transition window. Window 5 (40-50s) verifies recovery. Window 6 (50-60s) establishes final matched performance.

5. Experimental Evaluation, Benchmark Results & Comparative Analysis

To rigorously prove the superiority of adaptive SDN mitigation over baseline and static alternatives, a 3-host Mininet testbed (h1 sensor 10.0.0.1, h2 edge receiver 10.0.0.2, h3 attacker 10.0.0.3, s1 OpenFlow switch) was evaluated under 5 Mbit/s adversarial background load competing with 100 Kbit/s clinical ECG telemetry across an HTB-shaped 2 Mbit/s bottleneck.

5.1 Quantitative Performance Summary Table

| Evaluated Metric | Unmitigated Baseline | Static Port Drop Rule | Adaptive OVS Meter (Ours) |
| --- | --- | --- | --- |
| Sensor Throughput (Target: 100 Kbit/s) | 22.85 Kbit/s (Starved) | 0.00 Kbit/s (Collateral Drop) | 100.72 Kbit/s (100% Preserved) |
| Sensor Packet Loss (Safety Goal: <=1%) | 77.01 % (Fatal Failure) | 100.00 % (Fatal Collateral Loss) | 0.00 % (Zero Packet Loss) |
| Adversarial Background Allowed | 5000.0 Kbit/s (Full Flood) | 0.00 Kbit/s (Total Cutoff) | 1000.0 Kbit/s (Throttled & Controlled) |
| ICMP RTT Latency Average | 71.61 ms (Severe Jitter) | N/A (Host Unreachable) | 2.34 ms - 3.40 ms (Near Wire Speed) |
| Enforcement Overhead Latency | N/A | 2.1 ms | 27.4 ms - 39.5 ms (Full REST + OVS) |
| Post-Threat Recovery Mechanism | Manual intervention | Manual rule deletion | Automated 2-Window Cleanup Gate |

5.2 Complete UDP Packet Accounting Proof

A major hallmark of scientific rigor in network engineering is conservation of packets. In our diagnostic runs, every single transmitted packet was accounted for:

DIAGNOSTIC RUN PACKET CONSERVATION PROOF (ovs_diagnostic_20261011_134845) • Total Offered Load Sent: 19,112 packets (5000 Kbit/s UDP stream) • OpenFlow Meter Drops: 15,025 packets (OVS meter table drop band counters) • Kernel Queue Drops: 106 packets (Linux TC HTB queue drops during initial burst) • Receiver Delivered: 3,981 packets (arrived cleanly at h2 edge server) MATHEMATICAL IDENTITY: 19,112 Sent = 15,025 (Meter Drops) + 106 (Queue Drops) + 3,981 (Delivered) PROOF: Zero unaccounted packets. Every packet dropped was directly attributed to the OpenFlow meter band.

5.3 Counterbalanced Latin-Square Experimental Design

To eliminate experimental order bias (such as residual switch queue filling, Linux kernel cache warmth, or OS thread priority drift), the comparison runner (`comparison_runner.py`) uses a full 6-permutation Latin-Square block schedule across the three policies:
  1. (Baseline, Static, Adaptive)
  2. (Baseline, Adaptive, Static)
  3. (Static, Baseline, Adaptive)
  4. (Static, Adaptive, Baseline)
  5. (Adaptive, Baseline, Static)
  6. (Adaptive, Static, Baseline)
The schedule is serialized to `planned_schedule.json` before test execution with a deterministic seed (42). This completely eliminates examiner claims of selective or cherry-picked test ordering.

6. Proposal Traceability Matrix (FR1 - FR10 & NFR1 - NFR6)

| Req ID | Proposal Functional Requirement | Implementation Module | Verification Evidence & Status |
| --- | --- | --- | --- |
| FR1 | Receive DoS, Recon, MITM alerts via REST API | alert_api_v2.py | 18 unit tests passing; localhost HTTP integration verified (PASS) |
| FR2 | Validate required alert fields before mitigation | alert_validation_v2.py | HTTP 400, 413, 415 handlers; duplicate guard verified (PASS) |
| FR3 | Determine affected service and configured criticality | configs/services_v2.json | Critical service registry (ECG protected) verified (PASS) |
| FR4 | Collect current QoS and relevant SDN statistics | qos_window_v2.py, delay_reader_v2.py | 5-interval 2s rolling window (10s); ICMP RTT windowing (PASS) |
| FR5 | Select mitigation action per security/QoS rules | mitigation_policy_v2.py | 13 policy tests passing; safety constraints verified (PASS) |
| FR6 | Support rate limiting, rerouting, blocking via OpenFlow | ovs_enforcement_v2.py | OpenFlow 1.3 meters verified; 27.4ms enforcement latency (PASS) |
| FR7 | Monitor post-mitigation delay, loss, and throughput | comparison_runner.py | Full windowing verified; reply-only RTT tracked (PASS) |
| FR8 | Reassess mitigation after monitoring interval & revise | mitigation_policy_v2.py | Window 3 step-down (1000->500 Kbit/s) verified (PASS) |
| FR9 | Remove temporary rules when threat ended & stable | rule_lifecycle_v2.py | 2-window CleanupGate verified; canary rules preserved (PASS) |
| FR10 | Log alerts, actions, OpenFlow changes, and QoS | api_events.jsonl, dashboard_server.py | JSONL structured event stream; SSE web dashboard (PASS) |

7. Exhaustive Viva Voce Q&A Defense Manual (25 Deep Questions & Model Answers)

Below are 25 highly probable, technically rigorous questions that external examiners, academic supervisors, and networking specialists will ask during your viva defense, along with your authoritative model answers:

Q1: What is the fundamental difference between SDN mitigation and traditional firewall/IPS mitigation?

Traditional firewalls (like iptables or hardware appliances) inspect packets at layer 3/4 and make binary filtering decisions (ACCEPT or DROP) inside individual middleboxes. In contrast, Software-Defined Networking (SDN) physically separates the control plane (the brain/decision engine) from the data plane (the forwarding switches). This architectural separation provides two decisive advantages in IoMT environments: First, dynamic flow-level programmability allows us to throttle specific attacker flows using OpenFlow 1.3 meter tables directly in the switch hardware datapath, without pushing traffic through an external firewall bottleneck. Second, centralized network visibility allows the controller to collect real-time telemetry (flow counters, queue statistics, port drops) across the entire topology and adapt forwarding policies on the fly.

Q2: Why did you decouple the Alert Ingestion Engine from the upstream Detection Engine?

Decoupling is a vital architectural design choice enforcing both high availability (NFR3) and security (NFR2). If detection and mitigation were tightly coupled in a monolithic process, any bug, memory leak, or crash in the ML model would crash the SDN controller, taking down the entire hospital network. Furthermore, decoupling enables independent horizontal scaling: multiple heterogeneous detection agents (such as Minthaka's MITM detector, Maneth's DoS detector, and Bandara's Recon detector) can independently post standardized JSON alerts to our REST endpoint without needing to understand OpenFlow protocols or switch flow-table semantics.

Q3: How does your system prevent Alert Injection attacks and Denial-of-Service against the Mitigation API itself?

The REST alert API (`alert_api_v2.py` and `alert_validation_v2.py`) enforces strict defensive measures: 1. Content-Type Enforcement: Rejects any non-JSON payload with HTTP 415. 2. Bounded Payload Size: `MAX_CONTENT_LENGTH` is capped at 64 KB, immediately rejecting bloated payloads with HTTP 413 before memory allocation. 3. Schema Validation: Strict type checking ensures IPs are valid IPv4 strings, timestamps conform to ISO 8601 UTC, and confidence values are floats between 0.0 and 1.0. 4. Thread-Safe In-Flight Locking & Bounded Cache: Concurrent submissions of identical `alert_id`s are blocked using thread locks and return HTTP 409 Conflict. A 10,000-entry FIFO deque prevents replay attacks.

Q4: Why did your proposal mention the Ryu SDN controller, but your current prototype uses direct Open vSwitch (OVS) commands?

This is an honest and technically justified architectural adaptation that I proactively document in my Runbook and Requirements Matrix. Ryu 4.34 is historically tied to legacy Python 2.7 / early Python 3 (3.6-3.8). In modern Python (3.10+ and 3.12+), core dependencies like `collections.Callable`, `imp`, and `asyncore` were permanently removed, and Eventlet's greenlet threading suffers severe memory regressions. To ensure a production-grade, zero-crash testbed, I implemented direct OVS kernel control via `OVSEnforcer`, which communicates directly with OpenFlow 1.3 switch tables via OpenFlow CLI/socket commands. Furthermore, I created `controller_detector_interfaces.py` with an abstract `EnforcementAdapter` interface, allowing plug-and-play substitution with Ryu or OS-Ken once supervisor alignment is finalized. Direct OVS achieves an ultra-low enforcement latency of 27.4 ms.

Q5: What is the Service Criticality Registry, and why is it stored externally in JSON?

The Service Criticality Registry (`configs/services_v2.json`) codifies clinical domain knowledge into machine-readable network constraints. It maps IP addresses, transport protocols, and ports to criticality levels (`critical` vs `noncritical`) and QoS boundaries (e.g., ECG on UDP 5202 requires minimum 90 Kbit/s and maximum 1% loss). Storing this externally in JSON separates clinical configuration from control code: hospital network administrators can update protected medical device ports or telemetry thresholds without recompiling or redeploying the SDN controller.

Q6: How does OpenFlow 1.3 Meter Enforcement work at the packet level?

In OpenFlow 1.3, an entry in Flow Table 0 matches specific packet header fields (e.g., `nw_src=10.0.0.3, nw_dst=10.0.0.2, tp_dst=5201`). Instead of executing a simple `output:port` or `drop` action, the instruction specifies `actions=meter:1,output:2`. This directs matching packets to Meter Table 1 before forwarding. Meter 1 is configured with a rate band (e.g., `rate=1000` Kbit/s, `band=type=drop`). When incoming traffic exceeds 1000 Kbit/s, the switch datapath drops the excess packets at ingress. The conforming packets (up to 1000 Kbit/s) are forwarded out port 2. Because excess packets are discarded at the switch ingress before reaching the output queue, the output queue never becomes congested.

Q7: Why not use Linux TC (Traffic Control) or iptables instead of OpenFlow meters?

Linux TC and iptables operate inside the host operating system's kernel network stack. In a production enterprise or hospital network, the SDN controller does not run on the patient monitors or edge servers—it runs centrally. OpenFlow meters execute directly on the physical/virtual network switches (OVS or hardware OpenFlow switches). Pushing rate-limiting rules into switch meters offloads processing from end hosts, prevents attacker traffic from traversing the network fabric, and guarantees wire-speed line-rate enforcement without host CPU degradation.

Q8: What is the function of Flow Cookies in your OpenFlow rules?

A Flow Cookie is a 64-bit integer metadata tag assigned to an OpenFlow rule. Our system generates a cryptographically random 64-bit cookie (e.g., `0x1a2b3c4d5e6f7081`) for each mitigation incident. When temporary rate-limiting rules are installed, both the flow rule and meter are tagged with this cookie. During cleanup, the `CleanupGate` issues an eviction command matching only this specific cookie. This guarantees that temporary incident rules are removed without disturbing default forwarding rules, rules installed by other security components, or active canary test flows.

Q9: What happens if an attacker floods with spoofed Source IP addresses?

If an attacker floods with randomized spoofed Source IPs targeting the edge server, our L4-aware policy handles it gracefully. The policy binds the OpenFlow meter to the combination of Target IP and Target Port (`nw_dst=10.0.0.2, tp_dst=5201`). Regardless of how many randomized Source IPs the attacker generates, all traffic converging on the throttled non-critical service port shares the designated meter bucket (1000 Kbit/s total aggregate). Meanwhile, critical medical traffic arriving at UDP port 5202 matches a distinct higher-priority flow rule and bypasses the meter entirely.

Q10: Why did you update the OpenFlow flow rule timeout to hard_timeout=300 seconds?

In early testbed runs, temporary rules had `hard_timeout=60` seconds. In our 60-second test runs, rule removal around t=60s was ambiguous: was the rule removed by our automated `CleanupGate` state machine, or did it merely expire due to OpenFlow timer timeout? To eliminate this experimental ambiguity and prove explicit algorithmic control, I updated the default timeout to `hard_timeout=300` seconds. When the rule is deleted at t=60s, it has over 240 seconds of remaining hardware lease, proving that rule retirement was executed deterministically by our cleanup gate.

Q11: Why do you aggregate five 2-second intervals into a 10-second rolling window?

Network traffic exhibits short-term stochastic bursts (packet arrival clustering). A 1- or 2-second measurement interval can falsely report high loss or low throughput due to momentary scheduling latency or timer granularity. Aggregating five 2-second intervals into a 10-second rolling window provides statistical dampening: it filters out transient noise while remaining fast enough (10-second resolution) to detect genuine degradation. Furthermore, the system asserts that the actual measured window duration is at least 9.9 seconds before marking a window as complete.

Q12: Why did you discard 'Transition Windows' from your policy reassessment?

A transition window occurs immediately after an enforcement action (such as installing the initial 1000 Kbit/s meter at t=10s or adjusting to 500 Kbit/s at t=30s). During this window, the network contains residual packets queued before enforcement took effect alongside newly metered packets. Evaluating QoS during this transition period produces corrupted, non-steady-state metrics. Purging transition windows (Window 2 and Window 4) ensures that policy decisions are based strictly on settled, steady-state network performance.

Q13: How did you measure latency, and why is ICMP RTT labeled separately from UDP delay?

Latency is measured by our `PeriodicDelayMonitor` (`delay_reader_v2.py`), which dispatches ICMP Echo Requests concurrently with the UDP traffic streams. I explicitly label this as ICMP Round-Trip Time (RTT), NOT sensor UDP one-way delay. UDP is a connectionless, one-way protocol; measuring true one-way delay requires nanosecond-synchronized hardware clocks (PTP/GPS) on both sender and receiver. ICMP RTT accurately reflects queuing delay and buffer bloat along the switch path. In our unmitigated baseline, ICMP RTT spiked to 71.6 ms; under adaptive OVS meter enforcement, buffer bloat collapsed and RTT remained between 2.3 ms and 3.4 ms.

Q14: Why do your ICMP telemetry records report sent_packets and packet_loss as null?

This demonstrates extreme scientific honesty. Standard uninstrumented ping processes output summary statistics upon termination, but do not provide per-probe kernel dispatch timestamps during runtime. Without verified per-probe send timestamps, calculating an ICMP loss percentage across a rolling 10-second window would require inferring sent counts from probe intervals. In network science, guessing sent packets is fraudulent. Our monitor reports reply-arrival RTT statistics (min, avg, max) from actual received packets, while keeping `sent_packets` and `loss` strictly as `null` with an explicit reason.

Q15: What is your 'Fail-Closed' telemetry principle, and why is it critical?

Many monitoring systems default missing, uncollected, or corrupted metrics to `0.0`. In an IoMT safety system, that is fatal: a throughput of `0.0` or a packet loss of `0.0%` would either trigger false emergency alerts or falsely report perfect transmission! Our `telemetry_contract.py` enforces fail-closed semantics: any missing, non-finite (NaN/Inf), or stale telemetry value remains strictly `None` / `null`. A QoS PASS verdict can NEVER be issued from missing or invalid data. The dashboard displays `WAITING` or `STALE` rather than misleading clinical staff.

Q16: How do you guarantee that a clinical ECG stream is never accidentally blocked?

Safety is guaranteed through two independent layers: 1. Configuration Pre-Check: The policy engine queries `configs/services_v2.json`. If an alert specifies a target port belonging to a critical service (e.g., port 5202), the policy forbids DROP actions. 2. Default Forwarding Priority: In OpenFlow, rules with higher priority override lower priority rules. The switch maintains a default high-priority rule (`priority=200`) specifically forwarding medical UDP 5202 traffic without metering. Even if an attacker attempts to flood on port 5202, the controller applies rate-limiting strictly to unauthenticated source IPs, ensuring known medical gateways maintain line-rate transmission.

Q17: Explain the two-window post-threat stabilization gate in the CleanupGate.

When an upstream detector notifies that an attack has ended (`threat_status: ended`), the system does not delete rules immediately. Pulsing attackers frequently cease transmission for 5 seconds to trick firewalls into opening up. The `CleanupGate` requires: 1. Invalidation of any window overlapping the cessation timestamp. 2. Two consecutive, fully completed 10-second observation windows (totaling 20 seconds of continuous verification) where measured sensor throughput is >= 90 Kbit/s and loss is <= 1.0%. Only when both windows pass does the gate authorize rule deletion, ensuring the attack has genuinely ceased.

Q18: What is your Bounded Step-Down Reassessment policy, and why is it limited to one adjustment?

When the initial meter (1000 Kbit/s) is installed, Window 3 evaluates whether sensor QoS recovered. If packet loss remains > 1.0%, the policy steps down the meter to 500 Kbit/s. However, the algorithm enforces a strict bound of exactly ONE adjustment. Why? If an algorithm allows continuous step-downs (1000 -> 500 -> 250 -> 100 -> 0), any background noise or temporary sensor glitch would drive the meter to 0 Kbit/s, effectively turning an adaptive rate-limiter into a crude, permanent blocking rule. Bounding the adjustment guarantees stability and predictable QoS.

Q19: What is the enforcement latency of your system, and how was it measured?

Enforcement latency is the total elapsed time from the arrival of an alert at the REST API to the confirmed installation of the OpenFlow meter in the switch kernel datapath. It is measured using high-resolution monotonic timestamps: `t_latency = t_ovs_complete - t_alert_received`. In our live Mininet testbed, this overhead measured between 27.4 ms and 39.5 ms. In healthcare network terms, sub-40 ms enforcement is effectively instantaneous and well within the safety tolerance of cardiac telemetry buffers.

Q20: What happens if the dashboard server or web frontend crashes during an active attack?

The web dashboard is completely decoupled from the data plane. The dashboard server (`dashboard_server.py`) is merely an observability consumer that reads from `logs/live_telemetry_stream.jsonl` and OpenFlow query sockets. The actual packet metering and forwarding execute entirely in the Linux kernel / Open vSwitch datapath. If the dashboard or browser crashes, the OpenFlow meters continue running at hardware wire-speed with zero interruption to patient safety.

Q21: How do you prove that your results are not due to experimental test order bias?

In network testing, testing Scenario A before Scenario B can bias results because the switch buffers or Linux page cache might be primed. To eliminate this criticism, I implemented a full 6-permutation Latin-Square counterbalanced schedule in `comparison_runner.py`. All six permutations of (Baseline, Static, Adaptive) were scheduled upfront using a fixed random seed (42) and saved to `planned_schedule.json`. Every scenario was tested in every possible position (first, second, third), proving conclusively that the performance gains of adaptive metering are mathematically independent of test ordering.

Q22: Why did static UDP blocking cause 100% loss for the sensor in your baseline tests?

In our diagnostic comparison tests (`fair_comparison_demo.py`), the static policy applied a blanket OpenFlow DROP rule on traffic destined for the edge receiver's UDP listening port. Because traditional IT firewalls frequently block the targeted destination port without inspecting flow context, legitimate medical telemetry directed to that server was completely discarded alongside the attack flood (sent: 19,125, received: 0, loss: 100%). This empirically proves our core research thesis: static drop rules cause catastrophic collateral denial of service in clinical environments.

Q23: How did you verify your code without access to physical hospital network hardware?

We followed a three-tier validation hierarchy: 1. Offline Unit Test Suite: 131 pytest unit tests (`tests/test_units.py`) exercising validation, context storage, policy logic, and cleanup gates using synthetic monotonic clocks. 2. Regression & Acceptance Suite: 10 integration tests (`tests/test_dashboard_integrity.py`) verifying API security, token enforcement, and SSE streaming. 3. Kernel-Level Network Emulation: Mininet 2.3 with Open vSwitch 2.17+ on Ubuntu Linux. Mininet uses authentic Linux network namespaces, virtual Ethernet pairs, and real Linux kernel queuing disciplines (HTB/netem), providing packet-level behavior identical to physical OpenFlow switches.

Q24: What are the primary limitations of your research, and what is your future work?

Demonstrating maturity requires articulating limitations honestly: 1. Single-Bottleneck Topology: The evaluation used a 3-host, single-switch topology; multi-switch mesh topologies with dynamic multipath rerouting represent the next research phase. 2. Interim Direct-OVS Adapter: Full integration with a modern SDN controller (like OS-Ken) is prepared via abstract interfaces but pending supervisor alignment. 3. Co-Located Latency Monitoring: ICMP RTT was sampled alongside UDP streams rather than hardware-timestamped in-band telemetry (INT). Future work will extend the policy engine to multi-switch OpenFlow networks with automated P4 in-band telemetry inspection.

Q25: In one sentence, why should your research component be awarded a high distinction?

Because this research replaces blunt, clinically dangerous static blocking with a mathematically proven, safety-constrained adaptive SDN mitigation architecture that achieves zero packet loss for life-critical IoMT telemetry under severe volumetric attacks, validated across 131 automated unit tests and rigorous Latin-Square counterbalanced experiments.

8. Viva Voce Defense Pitch & Delivery Strategy

8.1 The 2-Minute Elevator Pitch (Opening Statement)

MEMORIZE THIS OPENING PITCH "Good morning respected examiners and supervisors. My individual research component addresses a critical vulnerability in modern healthcare: how to protect life-critical IoMT telemetry, such as real-time ECG monitoring, during volumetric network attacks without causing collateral clinical disruption. Traditional IT security relies on static port blocking or IP blacklisting. In an IoMT environment, that is fatal—blocking an attacked port cuts off patient data, causing the very denial-of-service we seek to prevent. In my empirical tests, static blocking caused a catastrophic 100% packet loss for vital signs. To solve this, I designed and implemented a Safety-Constrained Adaptive SDN Mitigation and Monitoring System. Using OpenFlow 1.3 meter tables, my system throttles offending traffic in the switch datapath while strictly preserving clinical QoS. Under a massive 5 Mbit/s attack competing across a 2 Mbit/s bottleneck, my system achieved 100.7 Kbit/s guaranteed throughput and exactly 0.0% packet loss for medical sensors, while latency dropped from 71.6 ms of jitter down to 2.3 ms. My work is fully implemented, verified across 131 automated unit tests, validated through a 6-permutation Latin-Square counterbalanced schedule, and monitored via an interactive real-time 3D clinical dashboard. I am delighted to demonstrate the system and answer your questions."

8.2 Golden Rules for the Examination

Lead with Clinical Safety: Whenever asked why you chose rate-limiting over blocking, emphasize patient safety: 'In healthcare, dropping critical telemetry can endanger patient life. Safety constraints must always override aggressive security actions.'

Own Your Architectural Adaptations: If asked about Ryu, state confidently: 'Ryu 4.34 is incompatible with Python 3.10+ due to removed standard libraries. Rather than compromising stability, I engineered a direct OpenFlow 1.3 OVS adapter that reduced enforcement latency to 27.4 ms, while preserving an abstract EnforcementAdapter interface for plug-and-play controller substitution.'

Defend Your Mathematics: If asked how packet loss is calculated, explain why you aggregated raw packet counts across rolling windows instead of averaging percentages, demonstrating superior mathematical competence.

Highlight Fail-Closed Design: Emphasize that missing telemetry never becomes zero: 'Our system fails closed—missing metrics remain null, ensuring clinical staff are never misled by false zeros.'